# DI Turnover Specification

Defines the cut, sources, references, exceptions, and zero-frame conform check delivered to the DI.

How to use: Accept or replace each recommended value. Complete the amber column, issue the document to the named recipients, and retain the approved revision.

## Document control

| Field or test | Recommended default or requirement | Production value or result |
| --- | --- | --- |
| Show | Full project title and short production code |  |
| Spec ID | PROJECT-DI-TURNOVER-SPEC |  |
| Revision | v001. Increment for every issued change. |  |
| Issued | ISO date and time with time zone |  |
| Effective | Immediately unless a future handoff is named |  |
| Supersedes | None for the first issue. Otherwise, name the prior revision. |  |
| Prepared by | Named person, role, company, and contact |  |
| Approved by | Named production and technical approvers |  |
| Issued to | Named people and companies |  |
| Acknowledgment record | Recipient, date, revision received, and exceptions |  |
| Authoritative location | One production-controlled file or record |  |

## Specification

| Field or test | Recommended default or requirement | Production value or result |
| --- | --- | --- |
| Cut identifier | Project, editorial revision, lock state, date, and runtime |  |
| Project timebase | Match the image-pipeline and editorial project specifications |  |
| Source-rate and ramp report | List off-speed, mixed-rate, interpreted, timewarped, and speed-ramped sources. Record sensor, playback, and timeline rates. |  |
| Reel or longplay structure | One longplay for a simple single-operator finish. Use versioned reels when picture, sound, VFX, or mastering benefits. |  |
| Reel boundaries | Record timecode hours, frame ranges, and any later rebalance |  |
| Leaders and pops | Eight-second digital head leader and a one-frame two-pop exactly two seconds before first frame of action when reels are used |  |
| Interchange file | Supply the NLE's strongest supported interchange plus a flattened EDL |  |
| Offline reference | QuickTime with matching timecode, source and record burn-ins, framing, and guide audio |  |
| Camera-original delivery | Never the production's only copy. Supply camera originals or checksummed consolidated media with a file manifest. |  |
| Consolidation handles | Record the required source handles for non-VFX media |  |
| VFX status list | Shot ID, intended revision, state, expected delivery, and current DI revision |  |
| VFX delivery | Full-quality finals, matching QuickTimes, manifest, and approval record |  |
| Titles and graphics | Source project, fonts, reference, alpha, color encoding, and final or temporary state |  |
| Optical-effects list | Simple and complex opticals, responsible party, source, reference, and approval state |  |
| Color metadata | Final dailies CDL values, Show LUT or creative transform, stills, ALE, and exceptions |  |
| Audio reference | Current guide mix, two-pop where required, track layout, and sound-turnover ID |  |
| Post-lock change list | One versioned list with old and new timecode, source, action, affected VFX and sound, and responsible approver |  |
| Re-turnover procedure | Issue a new cut ID, interchange, reference, change list, and affected media package |  |
| Conform-check method | Assistant editor compares DI and editorial by difference, checkerboard, or synchronized A/B with matching burn-ins |  |
| Conform tolerance | Zero frames and no unapproved framing, opacity, speed, effect, or source change |  |
| Conform approval | Assistant editor, result, date, discrepancy list, and corrected-check revision |  |